GUARDABARROS  TRASERO  ADAPTABLE A SUZUKI DL650 V-STROM 04’-11’

REAR MUDGUARD  ADAPTABLE TO SUZUKI  DL650 V-STROM 04’-11’

REF:4411

INSTRUCCIONES DE MONTAJE

- Desmontar el cubrecadenas original y el protector de suspensión.

- Montar la brida suministrada, por debajo de la guía de freno (4) tal y como muestra el croquis adjunto(se suministra espuma adhesiva para colocar en el interior de la brida, como protección del basculante). Usar el tornillo M6x15 hexagonal en la posición 2 y el tornillo M6x20 hexagonal en la posición 3, junto con las tuercas M6 grafiladas.

- Colocar los silenblocks en los taladros del guardabarros.  Para el punto de fijación próximo al amortiguador usar el  agujero “A” para versión2007 y el agujero “B” para las anteriores.

- Montar el guardabarros usando los tornillos allen de M6x15 en los anclajes de fijación del cubrecadenas y del protector de suspensión. En el herraje usar el tornillo allen de M6x20 juntamente con la tuerca de M6 grafilada.

- Ajustar el guardabarros respecto al neumático mediante la regulación que ofrece el herraje y apretar el conjunto.

- Vigilar periódicamente el ajuste y apriete de las fijaciones.

- En caso de que su motocicleta monte otros accesorios, comprobar que no interfieran con el guardabarros.

INSTRUCTIONS TO ASSEMBLE

- Dismount the original chainguard and shock absorber protector.

- Mount the bridle supplied underneath the brake guide (4) as the figure shows (it is supplied adhesive foam to place in the interior of the bridle as a proctetion of the swing arm). Use the M6x15 hexagon screw on position 2, and the M6x20 hexagon screw on position 3, jointly with M6 nuts.

- Place the well nuts in the hugger drills. For the fixation point close to the shock absorber use the hole “A” for the 2007 model, and the the hole “B” for previous models.

- Mount the hugger using the M6x15 allen screws on the fixation points of the chainguard and shock absorber protector. From the hardware, use the M6x20 allen screw jointly with the M6 nut.

- Adjust the mudguard with the tire through the regulation that the hardware offers, and tighten them all.

- Check them periodically, and tighten the fixations.

- In case your motorcycle has other accessories, observe that it does not interfere with the hugger.

Material sumistrado para el montaje en la motocicleta:

Material supplied to assemble the hugger:

| DENOMINACIÓN |  | CANTIDAD |
| --- | --- | --- |
| ## Tornillo AllenM6x20 negro DIN1001 | M6x20 allen screws | 1 |
| Tornillo Allen M6x15 negro DIN1001 | M6x15 allen screws | 3 |
| ## Tornillo Hexagonal M6x15 blanco DIN933 | M6x15 Hexagon screws | 1 |
| Tornillo Hexagonal M6x20 blanco DIN933 | M6x20 Hexagon screws | 1 |
| Tuercas grafiladas M6 DIN6923 | M6 nuts | 3 |
| Herraje | Iron fittings | 1 |
| Silenblocks | Well nuts | 4 |
| Espuma adhesiva | ### Foam Sticker | 1 |